NOTIFICATION REQUIREMENTS FOR POLLUTION AND SECURITY INCIDENTS

- A reportable pollution incident may involve either a discharge of oil or a release of a hazardous substance; a discharge means any spilling, leaking, pumping, pouring, emitting, emptying or dumping of oil that creates sheen, film, sludge, emulsion or discoloration in or on a U.S. navigable water way.  For chemicals under CERCLA a release is defined as any spilling, leaking, pumping, pouring, emitting, emptying, discharging, injecting, escaping, leaching, dumping, or disposing of a hazardous substance into the environment defined as air, land, or water.  A discharge (or potential) of oil must be reported to the National Response Center (NRC) every time and a release of hazardous substance must be reported when it meets or exceeds the reportable quantity (RQ) listed in 40 CFR 117.3.  Additionally, if you are required to have a Spill, Prevention and Countermeasures Plan (SPCC) and the amount of oil spilled to water is more than 1,000 gallons or more than 42 gallons on two different occasions within a 12-month period, then you must also notify your EPA Regional office in writing.

- A reportable security incident may involve suspicious activity, a breach of security or a transportation security incident (TSI). A breach of security is an incident that has not resulted in a TSI in which security measures were circumvented, eluded or violated. A TSI is an incident resulting in a significant loss of life, environmental damage, transportation disruption, or economic impact. Suspicious activity and breaches of security shall be reported to the NRC in accordance with 33 CFR 101.305(a) & (b) and a TSI shall be reported to the local Captain of the Port and the NRC in accordance with 33 CFR 101.305(c) without delay.

- The Chemical Safety Board (CSB)'s reporting rule requires prompt reports to the CSB from owners or operators of facilities that experience an accidental release of a regulated substance or extremely hazardous substance that results in a death, serious injury or substantial property damage. This information must be reported within 8 hours of the accidental release and may be made to the CSB by email at report@csb.gov or by telephone at 202-261-7600.

- The number for the National Response Center (NRC) is 800-424-8802. Notifications may also be required to Texas agencies (800-832-8224: Texas) and local agencies.  When facility boats enter any security zone waterway, they must report to Vessel Traffic Service at USCG Sector Houston-Galveston at 281-464-4837.